Literature reviews for powerpoint

- Fault Diagnosis of Permanent Magnet Synchronous Motor of Coal Mine Belt Conveyor

Year: 2022

What was done

Fault diagnosis of PMSM using digital twin and ISSA-RF method

ISSA-RF model optimized with Random Forest decision trees for fault diagnosis.

Training subsets sampled, features selected to form Random Forest classification model.

How was it done

Improved Sparrow Search Algorithm optimized Random Forest for PMSM fault diagnosis.

Hybrid ISSA addressed population distribution, diversity, and convergence speed issues.

Digital twin model constructed for real-time mapping of physical-virtual entities.

Merits and Demerits

Merits: ISSA-RF achieves 98.2% fault diagnosis accuracy.

Demerits: SSA algorithm is not as accurate as ISSA.

- Fault detection and classification of permanent magnet synchronous motor.

What was done

Proposes fault detection in variable load conditions using machine learning.

Algorithm tracks frequency orders associated with faults for diagnosis.

Features generated from tracking orders, torque, and speed features.

Support vector machine algorithm used for fault classification.

Validated method with experimental data for practical applications.

How was it done

- Algorithm tracked frequency orders from normalized order spectrum. [1]

- Fault features generated from tracking orders, torque, and speed features. [1]

- Support vector machine algorithm used for fault classification. [1]

- Experimental data validated the proposed fault detection and classification method.

Merits and Demerits

- Merits: High efficiency, power density, and reliability in wind turbines. [1]

- Demerits: Faults mainly detected in steady-state under constant loads and speeds.

- Fault Detection in Permanent Magnet Synchronous Motors using Machine Learning.

What was done

Detects Inter-Turn Short Faults in PMSMs using ML algorithms.

How was it done

- Developed ML models using experiential and simulated PMSM data. [1]

- Tested ML models like Random Forest, Gaussian SVM, KNN, and CNN. [1]

- Achieved over 95% accuracy in detecting ITSF in PMSM

Merits and demerits

- Detects Inter-Turn Short Faults in PMSMs with over 95% accuracy. [1]

- ML models can identify faults in PMSMs with three coils connected. [1]

- Demagnetization faults in PMSMs caused by high temperatures and short circuits

- Fault Diagnosis and Fault Frequency Determination of Permanent Magnet Synchronous Motor

Year: 2021

What was done

- Proposed CNN model diagnoses motor states with high accuracy. [1]

- Model extracts multiscale features and reduces training parameters effectively

- Proposed a 1D CNN for motor diagnosis using deep learning. [1]

- Reduced training parameters to solve overfitting. [1]

- Introduced class feature map for automatic frequency component determination.

How was it done

- Data collected on motor fault diagnosis platform with varying loads. [1]

- Signals preprocessed by zero-mean operation and FFT method.

- One-dimensional CNN for motor diagnosis with feature-extraction modules. [1]

- Weighted global average pooling to determine frequency components for classification. [2]

- Training times of different models and prediction speeds compared

Merits and demerits

Merits: Proposed model achieves 98.85% classification accuracy.

Demerits: Torque frequency information less influential for classification accuracy

- Faults and Diagnosis Methods of Permanent Magnet Synchronous Motors: A Review

Year: 2018

How was it done

Review of faults and diagnosis methods of Permanent Magnet Synchronous Motors

How was it done

Faults and diagnosis methods of PMSM were reviewed.

Methods included model-based diagnosis, signal processing, and data-driven algorithms.

Fault diagnosis methods include model-based, signal processing, and data-driven algorithms.

Sparse representation theory compresses signals for denoising and anti-interference.

SVM suitable for small sample classification, wavelet SVM for complex signals.

Signal processing methods: time domain, frequency domain, and time-frequency analysis.

Data-driven algorithms like PCA, ICA, neural network, SVM, and deep learning.

Merits and demerits

- The paper reviews faults and diagnosis methods of Permanent Magnet Synchronous Motors.[1]

- The faults include electrical, mechanical, and magnetic issues in PMSM.[1]

- Mechanical faults involve bearing faults, air gap eccentricity, and shaft bending.[2]

- Demagnetization fault in PMSM can be caused by various factors.

- Neural Network-Based Diagnostic Tool for Detecting Stator Inter-Turn Faults, 2021.

What was done

- Proposed neural network tool detects stator inter-turn faults accurately. [1]

- Achieved 93.125% accuracy in fault detection. [1]

- Developed FEM model for LSPMSM with stator inter-turn fault. [2]

- No control strategies employed in the research work.

How was it done

- Neural network-based diagnostic tool for stator inter-turn faults detection. [1]

- Features extracted from steady-state three-phase stator current signals used. [1]

- Finite element model developed for interior mount LSPMSM. [2]

- Simulation and experimental test results showed high accuracy in fault detection. [2]

- Neural network trained with different loading conditions and fault severities.

- Developed a feed-forward neural network diagnostic tool for fault detection. [1]

- Used statistical and frequency-based features extracted from stator current signals. [1]

- Tested motor with different numbers of shorted turns and loading conditions. [2]

- Validated results through experimental testing under healthy and faulty conditions.

Merits and demerits

- Proposed diagnostic tool accuracy: 93.125% in detecting faults. [1]

- LSPMSMs have high efficiency, power factor, and self-start capability. [2]

- Stator inter-turn faults account for 36% of motor faults. [3]

- Modeling LSPMSMs during faults is challenging due to differences in performance.